عطاء رقم 5/ل ر/2023

توريد وتركيب وتشغيل أجهزة مخبرية/ كلية العلوم

|  | Item | Quantity | Description |
| --- | --- | --- | --- |
|  | Benchtop instrument to identify bacterial and fungal species and antibiotics resistant profile (including all accessories, kits, software and PC) | 1 | Instrument to identify bacterial and fungal species Antibiotics resistant profile (MIC, resistant breakpoints based on CLSI, EUCAST, FDA). Fully automated and incubated system PC and software (if required) |
|  | Cabinet biological safety class 2 | 2 | Internal size 1300x500x600 mm HEPA filter 0.3 µm Vertical laminar negative pressure model With UV lamp With stand |
|  | 96 well plates spectrophotometer for fluorescence assay | 1 | Benchtop luminometer- fluorometer: 96 well plates All filters (UV/ BLUE/GREEN/RED/AFC) Shaker system Heating up to 45 ºC PC and software (if required) |
|  | Fluorescent microscope | 1 | Slide Triocular Filters: blue, green, red, violet and ultraviolet Camera 3 MP PC and software (if required) |
|  | Digital biological microscope | 1 | with a separate full HD screen. Data Output USB and HDMI Storage at least 128G. With a Brightness Adjustable Illumination system. Camera 3 MP and high Photo Resolution PC and software (if required) |
|  | tDCS brain stimulator | 1 | The device is a tDCS but possible to conduct it to ACS. AC/DC currents ONE channel stimulation is either anodal/cathodal. Stimulator should display the current dose, time duration, and frequency. Current intensity 0.1 -5mA Frequency phase free adjusted up to 200 Hz Duration of stimulation 5-40 minutes. |
|  | simNeuron: virtual physiology set for voltage and current measurement in neurons | 1 | It’s a program used for Virtual physiology for voltage and current measurement in neurons/ muscles/ hearts and vessels. Virtual physiology programs. Set contains; Sim-neuron, Sim-muscle; Sim-vessels, Sim-heart. |
|  | Chemistry analyzer (+ reagents) | 1 | fully-automated chemistry analyzer able to handle serum, plasma, urine, and cerebrospinal fluid samples with a clot detector and ISE module, and reusable Cuvettes Space Requirements: 90 cm (length) × 70 cm (depth) × 60 cm (height) Wight: maximum 150 kg Sample size: maximum of 40 μl, Operating system: fully automated Available tests: should include cardiac tests, LFT, KFT, Anemia profile, diabetic profile, electrolytes, pancreatic tests, and immune antibodies. Test Rate: minimum of 60 tests per hour Operating Temperature: 15~30°C Favorable conditions: RO unit included in the offer, multiple species testing (human, Rat, mouse) |
|  | Hormone analyzer (+reagents) | 1 | Space Requirements: maximum height (70cm), width (130cm), depth (80cm) Wight: maximum of 60 kg Sample size: single sample with a Maximum volume of 100 µl Operating system: fully automated Available tests: should include stat cardiac tests, coagulation profile, thyroid and fertility hormones, tumor markers, vitamins, Iron tests, covid -19, hepatitis, allergy, HIV, ToRC, EBV, Measles, Mumps and H. pylori antibodies or antigens. Test Rate: minimum of 30 tests per hour Operating Temperature: 15~35 °C Electrical requirements: Voltage: 100-240 VAC, Electrical consumption: 1.5 – 0.8 A, Frequency: 50-60Hz Power: 150 Watts, Heat emission: approximately 512 Btu/hr. Favorable conditions: RO unit included in the offer, multiple species testing (human, Rat, mouse) |

- تقدم العروض حسب الجدول التالي:

| رقم البند | الكمية | الوحدة | وصـــــــف المـــــــادة | الصانع | موديل | المنشأ | سعر الوحدة | السعر الاجمالي | مدة التسليم |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |